# Lunar Rover Simulation Setup Guide (ROS2 + Gazebo)

## WHAT YOU RECEIVED

You received:

- lunar_rover_project.tar.gz

This contains:

- rover simulation package

- moon world

- URDF/Xacro

- launch files

- configs

- built workspace

# STEP 1 — Install VirtualBox

Download:

- VirtualBox

- Ubuntu 22.04 ISO

Install Ubuntu in VirtualBox.

Recommended VM specs:

- RAM: 8 GB

- CPU: 4 cores

- Video Memory: 128 MB

- Enable 3D Acceleration

# STEP 2 — Install ROS2 Humble

Open Ubuntu terminal:

sudo apt update && sudo apt upgrade -y

sudo apt install software-properties-common -y

sudo add-apt-repository universe -y

sudo apt update

Add ROS2 repository:

sudo apt install curl -y

sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.asc \

-o /usr/share/keyrings/ros-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] \

http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" | \

sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

Install ROS2:

sudo apt update

sudo apt install ros-humble-desktop -y

Add ROS to bashrc:

echo "source /opt/ros/humble/setup.bash" >> ~/.bashrc

source ~/.bashrc

# STEP 3 — Install Gazebo + ROS packages

sudo apt install -y \

python3-colcon-common-extensions \

ros-humble-xacro \

ros-humble-robot-state-publisher \

ros-humble-joint-state-publisher \

ros-humble-ros-gz \

ros-humble-ros-gz-sim \

ros-humble-ros-gz-bridge

# STEP 4 — Copy Project

Place:
lunar_rover_project.tar.gz

inside Ubuntu home folder.

Extract:

tar -xzvf lunar_rover_project.tar.gz

# STEP 5 — Build Workspace

cd ~/ros2_ws

colcon build

Then:

source install/setup.bash

# STEP 6 — Run Moon World

gz sim ~/moon.world

Ignore warnings like:

- libEGL

- VMware

- Wayland

Those are normal in VirtualBox.

# STEP 7 — Spawn Rover

Open NEW terminal:

gz service -s /world/moon_world/create \

--reqtype gz.msgs.EntityFactory \

--reptype gz.msgs.Boolean \

--timeout 3000 \

--req 'sdf_filename: "/home/vboxuser/rover.urdf", name: "lunar_rover", pose: {position: {x: 0, y: 0, z: 5}}'

Rover should appear.

# STEP 8 — Move Rover

Open another terminal:

while true; do

gz topic -t /cmd_vel -m gz.msgs.Twist -p 'linear: {x: 2.0}'

sleep 0.1

done

Stop:
CTRL + C

# TURN ROVER

while true; do

gz topic -t /cmd_vel -m gz.msgs.Twist -p 'angular: {z: 0.5}'

sleep 0.1

done

# MOVE + TURN

while true; do

gz topic -t /cmd_vel -m gz.msgs.Twist -p 'linear: {x: 2.0} angular: {z: 0.3}'

sleep 0.1

done

# STOP ROVER

gz topic -t /cmd_vel -m gz.msgs.Twist -p 'linear: {x: 0.0} angular: {z: 0.0}'

# NOTES

- Use mouse to move camera.

- Press Play button if simulation pauses.

- Ignore VirtualBox graphics warnings.

- If rover disappears, respawn it.

Project includes:

- cinematic moon terrain

- rover physics

- differential drive

- Gazebo simulation

- ROS2 integration

Project Status:
WORKING ✅